ULD – UNIT LOAD DEVICE

Unloading LD3 containers from a Boeing 747

A unit load device (ULD), is a pallet or container used to load luggage, freight, and mail on wide-body aircraft and specific narrow-body aircraft. It allows a large quantity of cargo to be bundled into a single unit. Since this leads to fewer units to load, it saves ground crews time and effort and helps prevent delayed flights. Each ULD has its own packing list (or manifest) so that its contents can be tracked.

## Types

ULDs come in two forms: pallets and containers. ULD pallets are rugged sheets of aluminum with rims designed to lock onto cargo net lugs. ULD containers, also known as cans and pods, are closed containers made of aluminum or combination of aluminum (frame) and Lexan (walls), which, depending on the nature of the goods to be transported, may have built-in refrigeration units. Examples of common ULDs and their specifics are listed below.

| Volume indicated is internal volume. |  |  |  |
| --- | --- | --- | --- |
| Container type | Volume | Linear dimensions (base width / overall width × depth × height) | Remarks |
| LD1[1] | 4.90 m3 (173 cu ft) | 156 / 234 × 153 × 163 cm (61.5 / 92 × 60.4 × 64 in) | contoured, half width |
| LD2[2] | 3.40 m3 (120 cu ft) | 119 / 156 × 153 × 163 cm (47 / 61.5 × 60.4 × 64 in) | contoured, half width |
| LD3 | 4.50 m3 (159 cu ft) | 156 / 201 × 153 × 163 cm (61.5 / 79 × 60.4 × 64 in) | contoured, half width, dimension according to IATA; available at 45" height for loading on Airbus A320 family |
| LD6 | 8.95 m3 (316 cu ft) | 318 / 407 × 153 × 163 cm (125 / 160 × 60.4 × 64 in) | contoured, full width, equivalent to 2 LD3s |
| LD8 | 6.88 m3 (243 cu ft) | 244 / 318 × 153 × 163 cm (96 / 125 × 60.4 × 64 in) | contoured, full width, equivalent to 2 LD2s; DQF-prefix |
| LD11 | 7.16 m3 (253 cu ft) | 318 × 153 × 163 cm (125 × 60.4 × 64 in) | same as LD-6 but without contours; rectangular |
| Pallet type | Volume | Linear dimensions | Remarks |
| LD8 | 6.88 m3 (243 cu ft) | 153 × 244 cm (60 × 96 in) | same floor dimensions as container variant; FQA-prefix |
| LD11 | 7.16 m3 (253 cu ft) | 153 × 318 cm (60.4 × 125 in) | same floor dimensions as container variant; FLA- and PLA-prefixes |
| LD7 (2 pallet variants) | 10.8 m3 (381 cu ft) 11.52 m3 (407 cu ft) | 224 × 318 cm (88 × 125 in) 244 × 318 cm (96 × 125 in) | PAG- and P1P-prefixes PMC- and P6P-prefixes |

Pallet volumes shown are built 64 in tall for lower deck loading. Height limit for main deck depends on aircraft type.

## Aircraft compatibility

Cross-section of an Airbus 300 showing LD3 containers

LD3s, LD6s, and LD11s will fit 787s, 777s, 747s, MD-11s, Il-86s, Il-96s, L-1011s and all Airbus wide-bodies. The 767 uses the smaller LD2s and LD8s because of its narrower fuselage. The less common LD1 is designed specifically for the 747, but LD3s are more commonly used in its place because of ubiquity (they have the same floor dimensions such that one LD3 takes the place of one LD1). LD3s with reduced height (45" instead of 64") can also be loaded on the Airbus A320 family. LD7 pallets will fit 787s, 777s, 747s, late model 767s (with the big door), and Airbus wide-bodies.

Interchangeability of certain ULDs between LD3/6/11 aircraft and LD2/8 aircraft is possible when cargo needs to be quickly transferred to a connecting flight. Both LD2s and LD8s can be loaded in LD3/6/11 aircraft, but at the cost of using internal volume inefficiently (33 ft³ wasted per LD2). Only the LD3 of the LD3/6/11 family of ULDs can be loaded in a 767; it will occupy an entire row where two LD2s or one LD8 would otherwise have fit (90 ft³ wasted per LD3). Policies vary from airline to airline as to whether such transfers are allowed.

One of the design requirements of the 767's replacement, the 787, was for it to use the LD3/6/11 family of ULDs to solve the wasted volume issue.

### [edit] ULD capacity

Aircraft loads can consist of containers, pallets, or a mix of ULD types, depending on requirements. The table below indicates the maximum capacity of an aircraft for all-container and all-pallet configurations. In some aircraft the two types must be mixed as some compartments take only specific ULDs.

Container capacity of an aircraft is measured in positions. Each half-width container (LD1/LD2/LD3) in the aircraft it was designed for occupies one position. Typically, each row in a cargo compartment consists of two positions. Therefore, a full-width container (LD6/LD8/LD11) will take two positions. An LD6 or an LD11 can occupy the space of two LD3s. An LD8 takes the space of two LD2s.

Aircraft pallet capacity is measured by how many PMC-type LD7s (96" × 125") can be stored. These pallets occupy approximately three LD3 positions (it occupies two positions of one row and half of the two positions of the following row) or four LD2 positions. PMCs can only be loaded in cargo compartments with large doors designed to accept them (small door compartments are container only).

| A = Airbus; B = Boeing; L = Lockheed; MD = McDonnell-Douglas; F = freighter; ER = extended range; LR = long range |  |  |  |
| --- | --- | --- | --- |
| Aircraft | Max Container Cap. | Max Pallet Cap. | Remarks |
| B727-100F | none | 8 pallets*[3] | *88" × 125" pallets only; the 727 is a narrow-body |
| B727-200F | none | 12 pallets* | *88" × 125" pallets only; the 727 is a narrow-body |
| B727-200C (combi) | none | 11 pallets* | *88" × 125" pallets only; the 727 is a narrow-body |
| B757-200F | 13 SAAs* or 28 AYYs | 14 PAGs | *Position 14 is AYYs only |
| B747-100/200/300 | 30 LD1s[4] | 5 pallets + 14 LD1s, or 9 pallets |  |
| B747-400 | 32 LD1s[1] | 5 pallets + 16 LD1s, or 9 pallets + 2 LD1s |  |
| B747-400ER | 26 LD1s[1] | 4 pallets + 14 LD1s |  |
| B747-400F/ERF | 32 LD1s (lower deck) + 30 pallets (main deck)[1] |  | freighter aircraft, capacity includes all decks |
| B767-200 | 22 LD2s[2] | 3 pallets + 10 LD2s[2] |  |
| B767-300 | 30 LD2s[2] | 4 pallets + 14 LD2s[2] |  |
| B767-300ER | 30 LD2s[2] | 4 pallets + 14 LD2s[2] |  |
| B767-300F | 24 pallets* (main deck) + 30 LD2s (lower deck)[2] |  | *accepts 88" × 125" pallets only; freighter aircraft |
| B767-400ER | 38 LD2s[2] | 5 pallets + 18 LD2s |  |
| B777-200/200ER/200LR | 32 LD3s[5] | 10 pallets |  |
| B777F | 30 LD3s + 27 pallets | 37 pallets | freighter aircraft, capacity includes all decks |
| B777-300/300ER | 44 LD3s[5] | 14 pallets |  |
| B787-8/-3 | 28 LD3s | 9 pallets |  |
| B787-9 | 36 LD3s | 11 pallets |  |
| A300B2/B4 | 20 LD3s | ? |  |
| A300-600 | 22 LD3s | 4 pallets + 10 LD3s |  |
| A300-600F | 41 LD3s | 25 pallets | freighter aircraft, capacity includes all decks A300-600F deck layout |
| A310 | 14 LD3s | 3 pallets |  |
| A319 | 4 LD3-45W + 1 LD3/40 |  | Note LD3 height only 45"; add. cont. acc. IATA contour H |
| A320 | 7 LD3-45W |  | Note LD3 height only 45"; add. cont. acc. IATA contour H |
| A320PF | 10 pallets* (main deck) + 7 LD3-45W (lower deck) | 10 AAZ (main deck) + 7 LD3-45W (lower deck) | *accepts 88" × 125" pallets only; freighter aircraft equipped with fwd cargo 86×121" door |
| A321 | 10 LD3-45W |  | Note LD3 heigh only 45"; add. cont. acc. IATA contour H |
| A321PF | 13 pallets* (main deck) + 10 LD3-45W (lower deck) | 13 AAZ (main deck) + 10 LD3-45W (lower deck) | *accepts 88" × 125" pallets only; freighter aircraft equipped with fwd cargo 86×121" door |
| A330-200 | 23 pallets or 26 LD3s | 8 pallets + 2 LD3s |  |
| A330-200F | 9 AMA containers + 4 pallets (main deck) + 26 LD3 (lower deck) | 22 pallets (main deck) + 8 pallets + 2 LD3 (lower deck) | freighter aircraft, capacity includes all decks A330-200F deck layout |
| A330-300 | 32 LD3s | 11 pallets |  |
| A340-200 | 26 LD3s | 9 pallets |  |
| A340-300 | 32 LD3s | 11 pallets |  |
| A340-500 | 30 LD3s | 10 pallets |  |
| A340-600 | 42 LD3s | 14 pallets |  |
| A380-800 | 38 LD3s | 13 pallets |  |
| A380-800F | 59–71 LD3s | 66 pallets | freighter aircraft, capacity includes all decks A380-800F deck layout |
| MD-11 | 32 LD3s[6] | 26 pallets |  |
| L-1011 | 16 LD3s | none | all series except 500 (250/200/150/100/50/1 series) |
| L-1011-500 | 19 LD3s | 4 pallets* | *if equipped with fwd cargo 104" door |
| Il-86 | 16 LD3s | ? |  |
| Il-96 | 18 LD3s | ? |  |

Maximum capacity shown does not reflect weight restrictions.
Actual number of ULDs loaded may be lower if aircraft is at its weight limit.